INTERNATIONAL WEIGHTLIFTING FEDERATION

# Technology and Information System (TIS)

# Licensing of TIS Software and Hardware

# SUPPLIER PROPOSAL

Response to the IWF Request for Proposal dated 28 September 2026

Submission deadline: Saturday, 24 October 2026

# Cover Letter

To: International Weightlifting Federation

Email: iwf@iwf.sport

Subject: IWF TIS RFP - Proposals

Dear Evaluation Committee,

[SUPPLIER NAME] submits this proposal in response to the International Weightlifting Federation's Request for Proposal dated 28 September 2026 for the licensing, supply and operation of complete Technology and Information Systems for weightlifting competitions.

For [Claimed Profile], we propose a solution called OTIS+.  OTIS+ integrates the OWLCMS (Olympic Weightlifting Competition Management System) software with Blue Owl refereeing devices.   OTIS+ is a small extension of the existing OTIS kit that has successfully managed extremely large competitions (5 simultaneous platforms with 1600 athletes) in full compliance with the IWF TCRR.

[Address more advanced profiles]

The OWLCMS software has been in use since 2019 in its current form and has been used in over 107 countries.  It is the de facto standard for state, national and continental competitions in the Pan-American confederation and regional federations like the Nordic federation.  OWLCMS has historically been open-source, free software.  However, in the context of this proposal, OWLCMS refers to the same software core but under a commercial software license that includes support and additional modules enabled for advanced features (such as VPT or full IWF result books).  The free “community” version would remain available but would not claim IWF compliance.

Over 60 kits of Blue Owl refereeing devices explicitly designed to meet TCRR specifications have been sold worldwide.

We welcome the opportunity for certification of the software and device

Yours faithfully,

| Authorisation | Details |
| --- | --- |
| Name | [TO BE COMPLETED] |
| Title | [TO BE COMPLETED] |
| Signature | [TO BE COMPLETED] |
| Date | [TO BE COMPLETED] |

| Proposal information | Supplier response |
| --- | --- |
| Supplier legal name | [TO BE COMPLETED] |
| Product / solution name | [TO BE COMPLETED] |
| Exact baseline | [TO BE COMPLETED] |
| Profiles claimed | [TO BE COMPLETED] |
| Primary contact | [TO BE COMPLETED] |
| Email / telephone | [TO BE COMPLETED] |
| Proposal date | [TO BE COMPLETED] |
| Document version | Draft 0.1 |

# Revision Notes / TODO

Temporary working notes. Remove this section before final submission.

.

# A. Company

Legal name, registration, ownership/contact details, experience and references.

## A.1 Supplier Eligibility and Responsibilities

### A.1.1 Minimum supplier requirements

#### A.1.1.1 Legally established entity able to contract with IWF

[RFP §8.1] Legally established entity able to contract with IWF.

[To be completed]

#### A.1.1.2 Relevant experience delivering competition scoring/results or comparable mission-critical systems

[RFP §8.1] Relevant experience delivering competition scoring/results or comparable mission-critical systems.

[To be completed]

#### A.1.1.3 Ability to accept end-to-end responsibility for the complete system and declared third-party components.

[RFP §8.1] Ability to accept end-to-end responsibility for the complete system and declared third-party components.

[To be completed]

#### A.1.1.4 Qualified technical, competition-operations and support personnel

[RFP §8.1] Qualified technical, competition-operations and support personnel.

[To be completed]

#### A.1.1.5 International training, event support and incident-response capability

[RFP §8.1] International training, event support and incident-response capability.

[To be completed]

#### A.1.1.6 Ability to maintain the approved baseline for the license period

[RFP §8.1] Ability to maintain the approved baseline for the license period.

[To be completed]

# B. Product

Product name, architecture, supported profiles, capacities, deployment model and limitations.

## B.1 Solution Overview

The proposed solution is a field-proven combination of two technologies

- The OWLCMS software, used worldwide at the Continental, Regional, National and Local levels.  The software has been used in literally thousands of meets in 107 countries. OWLCMS is fully documented at https://owlcms.jflamy.dev .

- Blue Owl devices. Close to 70 kits of the devices proposed here have been sold.  The devices are explicitly designed to work with OWLCMS.  Some federations have deployed up to 6 platforms simultaneously with these devices. This proposal relieson the OTIS kit described at

Figure 1 and Figure 2 below provide a visual overview of the solution as proposed.

## B.2 Profiles

### B.2.1 M0 - Core Sport Conformance

[RFP §2.1] Complete sport logic, mandatory FOP controls, screens and official documents.

OWLCMS and Blue Owl are fully compliant with the TCRR as of October 2026.

### B.2.2 M1 - Licensed Operational Baseline

[RFP §2.1] M0 plus baseline control, audit, interfaces, offline operation, recovery, procedures, training, support and spares.

The system as delivered will meet all these requirements.  The Tracker element illustrated above provides a fully documented interface for all complementary systems that require one in more advanced profiles.

### B.2.3 M2 - Major Event & Broadcast

[RFP §2.1] M1 plus applicable VPT, public/live services, broadcast outputs, enhanced monitoring, high availability and full rehearsal.

Claimed: Yes. Video playback requires acquisition of additional video-capable equipment (laptops with video capture/gaming cards, cameras). OWLCMS has its own standalone VPT module that would be covered by this proposal. OWLCMS provides a set of TV Graphics for broadcasts, and its replay streams can be used as well. Supplier routinely does event-specific TV Graphics and has used the OWLCMS integration features to do so at large multisport games.

### B.2.4 M3 - Games & External Integration

[RFP §2.1] M2 plus approved Games/ORIS and programme-specific integrations.

Claimed: Yes. Supplier has previously integrated OWLCMS with the ODF (Olympic Data Format) for Games integration in the past (Islamic Solidarity Games 2025). The Tracker module uses the exact same protocol that is available for any external integration.  Full access to the competition data and to all competition events is provided over a secure connection, with TLS encryption (wss) supported.

Figure 1: Solution Overview

Figure 2: Detail: VPT and Interfaces

### B.2.5 MP - Multi-platform Overlay

[RFP §2.1] Simultaneous platform operation with separation and capacity evidence.

Claimed: Yes. OWLCMS has supported up to 6 platforms simultaneously with extremely low computer and network usage and 1600 athletes competing. The program is verified using a stress test designed explicitly for this purpose.

When used in this mode, a single instance of OWLCMS runs all platforms, and no integration step for results is required.  TCRR mandates that a lower entry group (e.g., group B) lifts before a higher one (e.g. group A); if this constraint is observed, all live results are correct.

## B.3 Licence certificate

The certificate will state supplier, product and exact baseline, approved profiles, capacities, interfaces, critical dependencies, limitations, deviations, validity and surveillance conditions.

### B.3.1 Supplier and product

[RFP §2.2] The licence certificate shall state the supplier and product.

The supplier of the product shall be [Supplier], who will act as integrator of the open-source OWLCMS version, of the Blue Owl OTIS kit to profile MP, and of the necessary additional components required to meet profile M3.

### B.3.2 Exact baseline

[RFP §2.2] The licence certificate shall state the exact baseline.

Any stable version of OWLCMS after version 69 is claimed to meet the current requirements and is fully compliant with TCRR as published at the time of this tender.  Any version of the Blue Owl software after version 3.0 meets the requirements. Final baseline will be confirmed after acceptance testing.

### B.3.3 Approved profiles and capacities

[RFP §2.2] The licence certificate shall state approved profiles and capacities.

M2 and MP profiles: The system has been tested to 6 concurrent platforms and 1600 participants, and can be stress-tested on demand. Examples of such very large mixed events include USAW Open 2024, USAW VWS Series 2025 and 2026, IMWA World Championships

M3 profile: [Supplier] integrated the OVR into the Islamic Solidarity Games 2025 central platform using ODF (Olympic Data Format) protocols.  [Supplier] will provide interfaces for the protocols used at given major events.

### B.3.4 Interfaces and critical dependencies

[RFP §2.2] The licence certificate shall state interfaces and critical dependencies.

The system has no critical dependencies. All communications use the standard HTTP and MQTT protocols.

The specific MQTT messages used for the devices are fully documented.

The protocol used to create additional interfaces for Data Exports, TV Graphics, etc. is public and has a published reference implementation.

### B.3.5 Limitations and deviations

[RFP §2.2] The licence certificate shall state limitations and deviations.

None known.

### B.3.6 Validity and surveillance conditions

[RFP §2.2] The licence certificate shall state validity and surveillance conditions.

Licensing authorizes the use of the TIS equipment and software package; the equipement and software may not be used if the user does not hold such a license.

IWF is expected to acquire TIS equipment and software licences from [Supplier] for IWF use at competitions that are directly under IWF authority.

A Federation licensing the TIS equipment and software package is expected to use it for the competitions that are under its own authority.  If the Federation wishes to lend the equipment to another federation, or to a constituent federation (such as a national federation loaning the equipment to a state federation), additional licensing fees may be required by the Supplier.

# C. Software

Modules, versions, rule-pack approach, database, interfaces, deployment and update model.

## C.1 Software, Data and Interfaces

### C.1.1 Software

#### C.1.1.1 Implement the effective IWF rule pack and support controlled updates

[RFP §6.1] Implement the effective IWF rule pack and support controlled updates.

The solution is tested against the TCRR rules using automated tests.  Any new release is tested against regressions.

#### C.1.1.2 Competition-critical functions shall not depend on unavailable internet, licence-server or payment services.

[RFP §6.1] Competition-critical functions shall not depend on unavailable internet, licence-server or payment services.

Once installed, the solution is completely autonomous and works with no internet connection whatsoever

#### C.1.1.3 Exact production version, configuration, database schema, rule pack and relevant assets shall be identifiable and reproducible.

[RFP §6.1] Exact production version, configuration, database schema, rule pack and relevant assets shall be identifiable and reproducible.

Releases are created by a fully repeatable automated build process, and under strict version control.  This applies both to OWLCMS and the Blue Owl components. Any past version can be retrieved and matched against its source code.

#### C.1.1.4 Installation and rollback shall be documented and repeatable

[RFP §6.1] Installation and rollback shall be documented and repeatable.

Installation and rollback are done with an installation program that allows rollbacks. Within a major version, rollback to a previous version is always possible.  Rollback to a prior major version is possible, obviously losing features introduced in that version.

#### C.1.1.5 Automatic updates shall not occur during an event freeze unless approved

[RFP §6.1] Automatic updates shall not occur during an event freeze unless approved.

The system does not perform any type of automatic update.  Operating system updates shall be managed by IWF or event personnel and not part of the product.

### C.1.2 Data integrity and authoritative state

#### C.1.2.1 Every sporting fact shall have one declared authoritative owner

[RFP §6.2] Every sporting fact shall have one declared authoritative owner.

Any change to the competition state is uniquely attributed to a human actor, to a device input, or to an automated calculation.  The audit log reflects this attribution.

#### C.1.2.2 Derived screens, feeds, documents and caches shall be traceable to the accepted state

[RFP §6.2] Derived screens, feeds, documents and caches shall be traceable to the accepted state.

All computed results are directly attributable to programmed rules based on the authoritative data.

#### C.1.2.3 Corrections shall be authorised, atomic, attributable and propagated

[RFP §6.2] Corrections shall be authorised, atomic, attributable and propagated.

Once the competition has started, all changes are meant to be made using the interactive interfaces or the explicit import features of the programs such that audit traces are kept.  Database state that does not correspond to audited actions is evidence of tampering. Should intentional exceptional corrective actions by the supplier requiring database queries or similar be necessary, the corresponding authorizations are expected to be managed separately as part of incident response.

#### C.1.2.4 Duplicate, lost, conflicting or out-of-order transactions shall be prevented or detected.

[RFP §6.2] Duplicate, lost, conflicting or out-of-order transactions shall be prevented or detected.

The system applies strict serialization of transactions. A relational database is used to enforce integrity constraints.

#### C.1.2.5 A complete machine-readable event export shall be available to IWF

[RFP §6.2] A complete machine-readable event export shall be available to IWF.

The database export contains all the event data. The audit logs are also machine-readable.

## C.2 Third-party dependencies

[RFP §6.4] All third-party software, libraries, services, licences and hardware necessary for operation shall be declared, including version, ownership, support status and cloud/public-internet dependency.

All third-party software, libraries, services, licences and hardware necessary for operation shall be declared, including version, ownership, support status and cloud/public-internet dependency.

All dependencies used in OWLCMS and Blue Owl are public and available on public servers.

### C.2.1 OWLCMS dependencies

OWLCMS is written in Java. The full dependencies for OWLCMS are listed in the following files.

- https://github.com/jflamy/owlcms4/blob/dev68/pom.xml

- https://github.com/jflamy/owlcms4/blob/dev68/owlcms/pom.xml

All the dependency files are found in the industry-standard “Maven Central” repository for dependencies.  Any past version of the software can be recreated as this site contains the full historical record of all the libraries.  The build process for OWLCMS is entirely automated and repeatable.

The main libraries in use are as follows

- The current long term support version of the Vaadin user interface toolkit; currently version 25.2.7

- Extremely stable version of the Hibernate data access library, version 5.6.15.Final

- Extremely stable version of the H2 relational database, version 2.1.124

- Extremely stable version of the Eclipse Jetty Web Services, version 12.0.4

- Extremely stable version of the Eclipse Paho MQTT library (version 1.2.4)

- Extremely stable version of the Google Guava library for event distribution (version 33.5.0)

- The very common Apache POI library for generating Excel spreadsheets (Version 4.1.2)

- The jxls library for creating templates of Excel files for report generation using Apache POI (version 3.0.0)

- The very common logback library for writing journals and logging information

- The standard libraries for Java 25

There are no other libraries that represent architectural commitments and would require any deep redesign. Versions labeled as Extremely Stable have been in use for over 5 years without need for a change other than security fixes made by the vendor that did not apply to our circumstances of use but were followed as a matter of principle.

The only library subject to likely future updates is the Vaadin user interface library, in order to benefit from better usability and make sure that the evolution of browsers/tablets/phones is accounted for.

### C.2.2 Blue Owl Dependencies

Blue Owl is a Node.js application.  As such it depends on

- The Firmata library called johnny5, version XYZ

- The node.js standard features

### C.2.3 Video Playback Dependencies

- The VPT module relies on the use of the ffmpeg software.  The current version of this software is typically used, in order to maximize the use of the graphical processors found on the video capture laptops.

- It is likely that users of the IWF-certified kit might chose a given video laptop over another to fit within the broadcasters’ preferred configuration. Ffmpeg is chosen exactly because it supports all known configurations as they become available.

# D. Hardware

Equipment list, specifications, quantities, spares and lifecycle/support information.

## D.1 Hardware and Field-of-Play Requirements

### D.1.1 Servers / control compute

[RFP §5] Controlled capacity for authoritative state and approved services, with spare/recovery path.

One main control computer with 8GB of memory and a 256GB disk is completely sufficient. ARM64 architecture as embodied in a Raspberry Pi5 is is demonstrated to handle 6 platforms with 1600 athletes with competition events taking place as fast as rules allow (stress test) and while providing all the requisite screens and attempt boards.

The proposed configuration includes a spare.

Furthermore computer or laptop running Windows, Linux or macOS can be substituted; the OWLCMS software runs identically on all these platforms, and the machine-readable database exports are readable on all.

### D.1.2 Operator stations

[RFP §5] Dedicated, role-controlled competition administration and operation positions.

Operator stations are only required to run one or two browsers and, for some, the Blue Owl software when devices are connected.  As a consequence, any machine with 4GB or more of memory can be used.  The proposed configuration uses 8GB Raspberry Pi500 that are easily replaceable, will be available as such for 15 years, and are designed to be used in difficult environments such as classrooms.

Keyboards with numeric pads will be provided for the Marshal and Data entry stations.

### D.1.3 Database / storage

[RFP §5] Protected authoritative storage, backup and tested recovery.

A fully relational database called H2 is used.  The volume of data used even in a very large competition is quite small, and H2 is extremely quick.  It has been used without a single attributable failure since 2009.  The actual H2 binary file can be kept, in addition to the fully machine readable exports.

The readable export is the primary means of backup and snapshot.  A readable snapshot can be obtained at any time from the administration or competition director stations, and can also be configured to be obtained on other stations using the tracker protocol.

### D.1.4 Referee controls

[RFP §5] Three independent red/white referee control devices.

The Blue Owl referee controls include the two buttons, a visible LED, a sound, and a vibration for warning the referees. Referee controllers are cabled in order to avoid any concern with batteries.

### D.1.5 Jury controls

[RFP §5] Individual devices for applicable Jury members plus President controls.

Jury controls for jury members are the same as the referee controls, but without the warning features.  Referee controllers can be used as backups for jury controllers (the warnings would simply not be emitted).

President controls are of two kinds.  The solution includes a physical keypad with all the required real-time referee decision display and the required jury member behaviour (green until all decisions visible)

A complementary tablet display is available in the kit; this enables the jury president to enter the reason for jury reversals.

### D.1.6 Timing

[RFP §5] Authoritative timer/controller with approved displays and audio.

Timing is done on the main server using the operating system timers.  Operating system timing resolution is 1 ms as used.  Propagation times of MQTT messages is under 10 ms.

Time is displayed on the attempt board and on an athlete-facing clock.

### D.1.7 Attempt boards

[RFP §5] Competition and warm-up presentation with mandatory athlete/attempt/weight/clock/decision data.

The attempt board is any standard FullHD TV with HDMI inputs.  The kit does not include the actual TV screens, that are assumed to be provided locally.

Two computers provide attempt board output.  One provides a standard black background output that is replicated on all TVs in the waiting, marshal and warmup areas. The other output is used for the attempt boards in the main room, where some styling (e.g., changing the background color) may have been done.

The kit includes splitters and extenders, assuming an attempt board is present in each of the waiting, marshal, and warm-up rooms, and that these areas are immediately behind the competition stage.  Further splitting or extension of the attempt board HDMI signal to reach multiple attempt boards is to be done using equipment and cabling provided locally.

For the main room, it is assumed that two attempt boards are present, one on each side of the stage.

### D.1.8 Scoreboards / RTDS

[RFP §5] Real-time competition and warm-up displays with correct ranking and state.

The situation for the scoreboards is the same as for attempt boards. There are two HDMI sources, one for the stage area, one for the back.  For the stage area, the signal is assumed to go to a single large LED scoreboard.

### D.1.9 VPT

[RFP §5] Approved cameras, recording/replay and Jury output for events requiring VPT.

The VPT system is under the full control of the competition staff, and the TCRR-mandated angles can be displayed in self-service mode by the jury if so required (or provided by the competition director).

As indicated in TCRR, the replay clips are accessible by the venue video team and broadcast teams. Such access will be controlled by networking equipment configurations.

The VPT system relies on four PTZ cameras.  Based on our experience in Continental competitions, we propose Tenveo PTZ 60fps cameras.  These cameras provide Power-over-Ethernet RTSP video feeds, HDMI, and compressed H.264 USB feeds.

OWLCMS has a VPT module that automatically clips and attributes the replays to the athlete lifting.  This module keeps a full indexed archive of all the lifts, at all angles.

The VPT will execute on a high-performance laptop that also meets the requirements for the vMix software.  If a client federation wants to add such additional video-processing capabilities, the proposed equipment will be sufficient for them to use their own license on the video-processing laptop.

### D.1.10 Network

[RFP §5] Documented, segmented, monitored event network with common time.

The networking configuration is based on switching.  Unmanaged switches are sufficient.  A mininimum of 4 POE (Power-over-Ethernet) ports is necessary to provide power to the networked PTZ cameras.  A 16-port switch per platform is provided.

An ASUS WiFi router is included.  The router is configured to provide DHCP addresses to all machines in the kit. The only machines whose addresses matter are the main OWLCMS server and the VPT laptop on each platform.  These will be set statically.  WiFi is not normally used for the competitions, only as a convenience during setup.  If federation personnel request access, it is preferred to loan them a network dongle.  This eliminates any concern about illicit WiFi access.  In the proposed configuration, the number of networking switches is kept to a minimum to prevent illicit connections.

All the machines on one platform are connected to the switch on that platform.  Use of a switch also ensures that video traffic from the cameras goes directly to the video equipment and does not touch other machines that don’t need it

OWLCMS networking traffic is minimal.  The application is routinely used in a cloud configuration with very remote servers and no perceptible delay.

### D.1.11 Public / broadcast gateways

[RFP §5] Controlled read-only path from accepted state to approved consumers.

The video laptop will host all the external interfaces.  In actual practice, sending information out to a games central is very low usage (and ODF protocols are push only).  OWLCMS can also publish to cloud-based scoreboards and other feature, but those are also push only.
In the past, outside broadcasters have used the OVR-provided screen layouts directly. This was accommodated by opening a port on the router to a specifically white-listed address.  This capability is generic with the equipment provided.

### D.1.12 Printing

[RFP §5] Reliable official documents with fallback.

A Windows laptop with Microsoft Office is used for data entry and report generation to maximize compatibility with whatever printer is made available.  OWLCMS produces documents in Excel format.  Documents should be exported as PDF in non-editable mode and kept as such for archival.

Start books, Result books, Refereeing schedules and similar sophisticated layouts are produced as PDF.

### D.1.13 UPS / protected power

[RFP §5] Protection and approved autonomy for critical equipment.

A UPS will be provided to protect the main server and the networking switches. The rest of the equipment, such as TVs, will likely go down during power issues, so there is no way to prevent outages other than a full generator (as used in Continental championships held in tropical countries subject to power fluctuations).

### D.1.14 Monitoring / logging

[RFP §5] Health, service, device, storage, time and stale-state monitoring.

The OWLCMS software has very low memory and disk I/O requirements. Use of the normal operating system indicators is therefore sufficient.  An administrative function does exist to query more precise runtime information and add it to the technical log, but only ever used during stress tests to confirm correct operation.

### D.1.15 Spares / recovery media

[RFP §5] Rapidly deployable replacements for critical equipment families.

All the stations only require the presence of a current browser (Chrome, Edge, Firefox or Safari), and for some of them, the ability to connect a USB device.   So essentially any laptop can be used, whether Windows, macOS, Linux.  Chromebooks can be used when no USB device is needed.

OWLCMS runs on Windows, macOS,

### D.1.16 FOP installation

[RFP §5] Safe, labelled cabling and equipment placement without obstruction or sightline conflicts.

[To be completed]

### D.1.17 Hardware inventory

[RFP §5.1] For each licensed event, the supplier shall maintain make/model, quantity, serial or asset ID, firmware, location, connections, power source, owner, health status, spare path and approved deviations.

For each licensed event, the supplier shall maintain make/model, quantity, serial or asset ID, firmware, location, connections, power source, owner, health status, spare path and approved deviations.

Supplier response
[TO BE COMPLETED]

# E. Competition functionality

Lifecycle, officiating, timing, displays, outputs, results and archive.

## E.1 Minimum Functional Requirements –

The supplier claims full compliance with all the requirements. Details are provided for each item.

### E.1.1 Data Intake

[RFP §3] Import and validate athlete, federation/NOC, official, event, venue and record data; stable IDs and error reporting.

Data is converted to an Excel-style sheet with the athlete identifier, name, and all registration data.  Competition information and dates are stored in the program database (all information is stored in a relational database).  The initial schedule is part of the data intake, and an initial assignment of athletes to sessions is done.

The input format handles multiple age groups and supports team allocation. The intake format can be exported, edited off-line, and re-ingested without loss of information.  The same applies to schedule changes and referee assignments that can be modified in bulk until stabilized.

### E.1.2 VFE / Technical Congress

[RFP §3] Corrections, approvals, random lots, grouping, timetable and Technical Official assignments.

The program prints VFE verification sheets that show exactly which athlete is in which category or categories.  There are also team membership teams to verify that all team allocations are correct.  These sheets can be signed by team representatives to facilitate the VFE process and reduce the number of errors, or used by the TIS team to do the capture and then get them signed.

### E.1.3 Start publication

[RFP §3] Approved Start List and controlled distribution/versioning.

OWLCMS as proposed is able to produce a fully compliant start book. Preliminary versions can be published to list participants, with subsequent editions adding the schedule and the officials assignments.  Once converted to the OWLCMS format, any number of records can be handled.
Field-of-play formats are also available (actual session assignments and schedule) for direct use by coaches and technical officials.

### E.1.4 Weigh-in

[RFP §3] Correct timing, bodyweight, first attempts, 20 kg checks, start numbers, signatures and closed snapshot.

OWLCMS produces field-tested Weigh-in forms to capture both the weight-on-scale and deducted weight.  All applicable rule are checked, including 20 kg and qualifying totals where applicable.  The form captures all the required signature.  Compliant athlete cards are likewise printed.  A special data entry mode is used for quick capture of weigh-in data.  A post-weigh-in snapshot of the database is taken.
Start numbers are cross-checked at the end of the weigh-in and checked against allocated bib numbers. A speaker introduction sheet for athletes and officials is produced, as are the empty protocols and jury member sheets.

### E.1.5 Snatch

[RFP §3] Calling order, declarations/changes, 60/120-second timing, referee decisions, records and live outputs.

Full compliance with the competition flow is attained.  All marshal data entry is immediately handled by the system and all lifting order or requested weights are instantly updated on all displays and scoreboards. All the normal timing decisions are automatic (the clock is automatically set to the right allocated time for each athlete, including on fast sequences of changes). Only explicit loading incidents require a reset.
All attempts are checked against the 20 kg rule and the required changes are explicitly visible to the marshal. Incomplete situations that do not fully restore the 20kg rule are explicitly visible.

### E.1.6 Break

[RFP §3] Controlled Snatch-to-Clean & Jerk transition and synchronized displays.

Default rules for longer breaks (few athletes) and shorter breaks (large sessions) are both available.  The session schedule also allows changing the break duration.  This can also be done after the session has started (under the assumption the meet director informs all concerned).
Transition from the snatch to the clean & jerk break at the end of the snatch is automatic.  A jury interruption or challenge on the last requires the time keeper starts the break once the situation has been resolved.

### E.1.7 Clean & Jerk

[RFP §3] Calling order, weight changes, 20 kg enforcement, totals, ranks and records.

The same completely automatic timing and weight change flow as in the snatch takes place.

### E.1.8 Review / correction

[RFP §3] STOP, Jury, challenge/VPT where applicable, additional attempts, correction, withdrawal and restart.

The jury can start a deliberation or accept a challenge using a physical jury keypad button or using a jury tablet.  A flashing STOP sign is shown to the athlete on the platform, and all attempt boards and displays indicate the interruption.  Transmission of the jury reversal code to the speaker can be done the traditional way (by the technical controller), or automatically when the jury president chooses the reversal code.  In either case, the jury or challenge decision is shown as the speaker announces it.
The system can optionally display jury decisions on the scoreboards and the attempt boards.  This capability is optional because it is not formally described in TCRR.

### E.1.9 Results approval

[RFP §3] Protocol reconciliation, signatures/status, results, classifications and records.

A session protocol sheet is available immediately after the session that contains all the requisite information and each occurrence of an improved record.  This is normally signed by the jury president.  After a session, any correction required can be made via the system to ensure an audit trail to match whatever decisions were written on the protocols.

### E.1.10 Ceremony / publication

[RFP §3] Medallists, public/broadcast outputs, communications and revised outputs.

The system distinguishes two types of scoreboards.  Public scoreboards are normally styled to match the event colors, and switch content during ceremonies.  OWLCMS will automatically switch during introductions.  During medal ceremonies, the technical table (or the speaker themselves) can change the display to show the medals. In multi-age or multi-federation championships, this allows correct control of what is shown to the public. If broadcasters elect to use the video layout screens as provided by OWLCMS, the changes are also automatic.

### E.1.11 Archive

[RFP §3] Final dataset, official outputs, logs, revisions, evidence and machine-readable export.

The system distinguishes two types of scoreboards.  Public scoreboards are normally styled to match the event colors, and switch content during ceremonies.  OWLCMS will automatically switch during introductions.  During medal ceremonies, the technical table (or the speaker themselves) can change the display to show the medals. In multi-age or multi-federation championships, this allows correct control of what is shown to the public. If broadcasters elect to use the video layout screens as provided by OWLCMS, the changes are also automatic.

### E.1.12 Core sporting logic

The core sporting logic is implemented is central logic called a “state automaton” that tracks all permissible actions. This logic in turn relies on computation rules (points, ranks, timing rules) that are automatically tested.  There are over 300 automated tests that exercise the core sporting logic.

#### E.1.12.1 Snatch followed by Clean & Jerk, maximum three attempts per lift, correct Total calculation.

[RFP §3.1] Snatch followed by Clean & Jerk, maximum three attempts per lift, correct Total calculation.

Fully compliant with TCRR

Evidence / reference: Explicitly in database schema, automated tests.

#### E.1.12.2 Whole-kilogram weights and applicable minimum starting weights

[RFP §3.1] Whole-kilogram weights and applicable minimum starting weights.

Fully compliant with TCRR

Evidence / reference: Explicitly in database schema, automated tests.

#### E.1.12.3 Correct calling order

[RFP §3.1] Correct calling order, including weight, attempt number, previous-attempt sequence and start number.

Fully compliant with TCRR

Evidence / reference: Automated tests for all the change sequences.

#### E.1.12.4 Permitted weight changes, final-call and consecutive-attempt rules

[RFP §3.1] Permitted weight changes, final-call and consecutive-attempt rules.

Fully compliant with TCRR

Evidence / reference: Automated tests for all the change sequences and for the two-minute situations. Systematic recalculation of lift order to detect illegal moving down, with automatic tests.

#### E.1.12.5 Withdrawals, no-lift/no-Total conditions, disqualification and additional attempts

[RFP §3.1] Withdrawals, no-lift/no-Total conditions, disqualification and additional attempts.

Fully compliant with TCRR

Evidence / reference: Automated tests of the total, disqualification and withdrawals.

#### E.1.12.6 Material corrections recalculate affected rankings, records and outputs

[RFP §3.1] Material corrections recalculate affected rankings, records and outputs.

Fully compliant with TCRR

Evidence / reference: Any change to the lifting data causes a recalculation of all affected items. Automated tests for the ranking and records calculations.

## E.2 Timing, Officiating and Outputs

### E.2.1 Timing

#### E.2.1.1 Authoritative electronic countdown in one-second increments, configurable up to at least 15 minutes.

[RFP §4.1] Authoritative electronic countdown in one-second increments, configurable up to at least 15 minutes.

Fully compliant with TCRR

Evidence / reference: Timekeeper controls all break durations, no limit on duration.

#### E.2.1.2 Automatic 60- and 120-second attempt clocks and applicable introduction/break countdowns

[RFP §4.1] Automatic 60- and 120-second attempt clocks and applicable introduction/break countdowns.

Fully compliant with TCRR

Evidence / reference: Clock ownership is automatic, no intervention required to set the clock to 1:00, 2:00 or to the time remaining for the athlete. Transition between countdowns (introduction to snatch for example) is automatic. Can be overridden as required.

#### E.2.1.3 Correct start, stop, restart and STOP behaviour according to the rules

[RFP §4.1] Correct start, stop, restart and STOP behaviour according to the rules.

Fully compliant with TCRR

Evidence / reference: As per TCRR

#### E.2.1.4 All connected displays use the same authoritative time state within the approved tolerance.

[RFP §4.1] All connected displays use the same authoritative time state within the approved tolerance.

Fully compliant with TCRR

Evidence / reference: Time is kept on the OWLCMS server; when sent to remote devices, actual universal time reference for timer termination so the device can compensate for eventual small clock drift. All devices are time-synchronized.

#### E.2.1.5 Time corrections are authorised, recorded and auditable

[RFP §4.1] Time corrections are authorised, recorded and auditable.

Fully compliant with TCRR

Evidence / reference: The displays are NTP-synchronized at the operating system level by default. Verifying time synchronization is part of the technical setup.

### E.2.2 Referee and Jury

#### E.2.2.1 Three independent referee control boxes with white/red decisions and required indicators.

[RFP §4.2] Three independent referee control boxes with white/red decisions and required indicators.

Fully compliant with TCRR

Evidence / reference: Nemikor referee buttons have visual indicator, sound, and vibration.

#### E.2.2.2 Correct majority/Down signal, reminder and reversal-window behaviour

[RFP §4.2] Correct majority/Down signal, reminder and reversal-window behaviour.

Fully compliant with TCRR

Evidence / reference: Implemented by software. Reversal rules are as per TCRR. Manual reversal of a decision following flags can easily be done by the technical table.

#### E.2.2.3 Jury visibility of individual referee decisions before public reveal

[RFP §4.2] Jury visibility of individual referee decisions before public reveal.

Fully compliant with TCRR

Evidence / reference: Live referee decisions and reversal are shown to the jury both on the keypad and on the jury console screen or tablet display if used.

#### E.2.2.4 Support for approved three- and five-member Jury configurations

[RFP §4.2] Support for approved three- and five-member Jury configurations.

Fully compliant with TCRR

Evidence / reference: Jury size is selectable,

#### E.2.2.5 Individual Jury decision controls and protected independence between members

[RFP §4.2] Individual Jury decision controls and protected independence between members.

Fully compliant with TCRR

Evidence / reference: Individual jury buttons are provided as per TCRR. Decision protection housings are not a TCRR requirement but will be addressed.

#### E.2.2.6 Material decisions and actions are attributable and time-stamped

[RFP §4.2] Material decisions and actions are attributable and time-stamped.

Fully compliant with TCRR

Evidence / reference: Full audit log of every single decision, timestamped to the millisecond, with the originating station or device.

### E.2.3 Screens and live presentation

The system is fully compliant with all these items. Details and examples for each item are given.

NOTE: For many of the items, there are several screens available. We illustrate the key items and do not claim to be exhaustive.

#### E.2.3.1 System health

[RFP §4.3] At minimum, the TIS shall provide System health.

A technical log is always available for detailed technical operations.   The memory, disk and networking usage are extremely low, the system gauges confirm this (the system never exceeds 20%, and is usually well below 10, even for extremely large meets).

#### E.2.3.2 Event setup

[RFP §4.3] At minimum, the TIS shall provide Event setup.

All competition information setup is done using interactive screens.  Green is used in the software to indicate common actions.

##### E.2.3.2.1 Competition Rules

The first tab concerns competition information and which federation homologates it.

Other Tabs contain the default rules for the meet, for example, what medals are given, how best athletes are determined, how team points are given, etc.

OWLCMS can also be used for local, national, regional and continental championships that have historical variations with full IWF rules.  The other tabs include all these options. The defaults comply with TCRR but acquiring organizations would nevertheless be able to use their licenses in different settings.

##### E.2.3.2.2 Lots/groups/timetable

[RFP §4.3] At minimum, the TIS shall provide lots/groups/timetable.

Normally an import from a competitions registration system is used to set up the initial schedule and assign athletes assigned to A/B/C… groups.  At the scale of IWF events, this is most likely done using output of the registration system in spreadsheet format.  This is then converted to fit the documented OWLCMS format (this has now become trivial with human assistance tools).  All data exchange formats for OWLCMS are public.

Once this is done all athletes can be edited in the program, and late entries added by hand if needed.

The registration screen allows modification of the allocated session, moving the athlete to another group, adjusting eligibility to multiple age groups as appropriate.

The other tabs on the registration form cover things like record eligibility (in joint jurisdiction championships, not all athletes are eligible to improve all records)

The session timetable is editable.

##### E.2.3.2.3 Assignments of Technical Officials

Add Timetable screen here

Adjustments to individual TO assignments can be done from here after the initial TO teams have been assigned.

#### E.2.3.3 Document Production

After registration and VFE, the relevant documents can be printed, such as the athlete cards and weigh-in forms. Documents are produced in Excel, and can therefore be easily converted to PDF if needed.  All the necessary documents are available (see the Documents section further down)

Groups of documents used together can also be created as one set (for example, the weigh-in sheet and the athlete cards), for one or more session (for example, for the next day).

#### E.2.3.4 Weigh-in

[RFP §4.3] At minimum, the TIS shall provide Weigh-in.

The weigh-in data entry form is intentionally the same format as the registration form. All relevant checks for the 20 kg rule are applied at data entry, in case they were missed by the weigh-in Tos.

After weigh-in, the page allows the creation of all the relevant forms for the speaker and for the jury.

#### E.2.3.5 Chief Marshal

[RFP §4.3] At minimum, the TIS shall provide Chief Marshal.

The marshal table has a station that shows all athletes (in start order or lifting order as preferred by the TO operating). Clicking on an athlete brings up a card that is laid out exactly as the athlete card, and automatically brings focus to the right cell, such that typing and hitting Enter can be used for fast data entry.

20kg rule violations and late changes are immediately made visible

#### E.2.3.6 Timekeeper

[RFP §4.3] At minimum, the TIS shall provide Timekeeper.

During the competition a tablet-friendly display is used, that can be used together with the Blue Owl buttons, as preferred.

The timekeeper can control the breaks (in addition to the main technical table or the speaker having the same feature depending on the skill and comfort levels). The delay can be set to a clock or duration as relevant for the type of break.

#### E.2.3.7 Speaker

In OWLCMS, a speaker page shows athletes in lifting order and displays all relevant notifications.  This makes it much easier for the speaker, who does not have to interpret in real time what is shown on the scoreboard.

#### E.2.3.8 Competition supervisor

[RFP §4.3] At minimum, the TIS shall provide Competition supervisor.

The competition supervisor screen is the as the speaker, with the addition of the decision lights that can be seen live as an option. This is because the switches between sessions, and breaks, and lift overrides after a late referee flag call can all be done from that screen.  Depending on competitions, the speaker screen will be made “passive” if the technical table assumes those tasks instead of an ITO1 comfortable with technology.

In addition, OWLCMS is launched from a control panel that gives additional options such as viewing the technical log going live.  The competition supervisor can watch the competition-level audit log as it is written.

#### E.2.3.9 Referee monitoring

[RFP §4.3] At minimum, the TIS shall provide Referee monitoring.

The competition director screen can see the live referee lights, as does the jury.  The audit log shows each individual decision, and each weight change from the marshals.

#### E.2.3.10 Jury control

[RFP §4.3] At minimum, the TIS shall provide Jury control.

The jury control can be done entirely using a Blue Owl jury control keypad

In the context of an IWF meet, it becomes desirable to accelerate the transfer of decisions from the jury to the speaker. To this effect, a tablet-style jury display is available that has the same features as the keypad

When this is used, the Speaker and Competition Supervisor screens get the reason immediately, and can resume the competition after announcing it.

#### E.2.3.11 Attempt boards

[RFP §4.3] At minimum, the TIS shall provide Attempt boards.

Attempt boards show all the information required by TCRR.  They can also display the flag of the country. If one or more record attempts take place the attempt boards show it as a rolling banner.

#### E.2.3.12 Scoreboards

[RFP §4.3] At minimum, the TIS shall provide Scoreboards.

Scoreboards contain all the requisite information. In addition, the top line contains the current attempt.
The bottom line contains additional information, such as the various championships, rank if successful, jury decisions etc.). This is the warmup room layout, that does not change during ceremonies so coaches can count attempts.

Scoreboard during session with previous leaders

In the main room, styling will be applied as required by the event, and the top row is used as a banner. A row at the bottom carries will carry the clock and other attempt information.
Main Room Scoreboard with previous leaders

Full-fledged video production companies can replace this scoreboard and the TV Graphics by using the extension capabilities provided by OWLCMS.  The extension protocol is documented with a reference implementation at https://github.com/owlcms/tracker-core .

#### E.2.3.13 RTDS/warm-up

[RFP §4.3] At minimum, the TIS shall provide RTDS/warm-up.

In OWLCMS all the screens are RTDS (Real-time Display System)

When decisions are given, they are shown both on the attempt board and on the scoreboards.

If jury decisions are being shown, these appear as well.  The bottom line then shows the athlete under review, whereas the top line shows the athlete getting ready (whose clock is not running).
Jury Decision for current athlete

#### E.2.3.14 STOP

[RFP §4.3] At minimum, the TIS shall provide STOP.

During deliberation, challenge, technical issue or other stoppage, the reason is shown

Flashing Stop

#### E.2.3.15 Records

[RFP §4.3] At minimum, the TIS shall provide Records.

Record Attempt Attempt Board

Record Attempt Scoreboard

#### E.2.3.16 Results

[RFP §4.3] At minimum, the TIS shall provide Results.

End of session rankings with medals

#### E.2.3.17 Other approved states

[RFP §4.3] At minimum, the TIS shall provide Other approved states.

### E.2.4 Official outputs

#### E.2.4.1 Start Lists

[RFP §4.4] At minimum, the TIS shall provide Start Lists.

#### E.2.4.2 Start List Package

[RFP §4.4] At minimum, the TIS shall provide Start List Package.

#### E.2.4.3 Weigh-in List

[RFP §4.4] At minimum, the TIS shall provide Weigh-in List.

#### E.2.4.4 Athlete's Cards

[RFP §4.4] At minimum, the TIS shall provide Athlete's Cards.

#### E.2.4.5 Empty Protocol

[RFP §4.4] At minimum, the TIS shall provide Empty Protocol.

#### E.2.4.6 Manual Protocol

[RFP §4.4] At minimum, the TIS shall provide Manual Protocol.

#### E.2.4.7 Protocol

[RFP §4.4] At minimum, the TIS shall provide Protocol.

#### E.2.4.8 Results

[RFP §4.4] At minimum, the TIS shall provide Results.

#### E.2.4.9 Team Classification/Placing

[RFP §4.4] At minimum, the TIS shall provide Team Classification/Placing.

#### E.2.4.10 Records

[RFP §4.4] At minimum, the TIS shall provide Records.

#### E.2.4.11 Final Results Package

[RFP §4.4] At minimum, the TIS shall provide Final Results Package.

#### E.2.4.12 Results Book where applicable

[RFP §4.4] At minimum, the TIS shall provide Results Book where applicable.

#### E.2.4.13 Machine-readable export

[RFP §4.4] At minimum, the TIS shall provide Machine-readable export.

#### E.2.4.14 Distribution records

[RFP §4.4] At minimum, the TIS shall provide Distribution records.

#### E.2.4.15 Final evidence/archive

[RFP §4.4] At minimum, the TIS shall provide Final evidence/archive.

Paper, PDF and machine-readable outputs are delivery channels; none may become an undocumented alternative source of sporting truth.

Supplier response
[TO BE COMPLETED]

# F. Security & resilience

Security architecture, access, offline operation, backup, recovery, failover and continuity.

## F.1 Security, Resilience and Business Continuity

### F.1.1 Security

OWLCMS meets all stated requirements.

#### F.1.1.1 Least-privilege access by role and device identity.

[RFP §7.1] Least-privilege access by role and device identity.

OWLCMS supports separate logins for every role, and where relevant, separate logins for each role and platform.  Devices are identified per platform and authenticated using the MQTT identity of the MQTT interface.

#### F.1.1.2 Separation of competition, device, public/display, database and management functions where applicable.

[RFP §7.1] Separation of competition, device, public/display, database and management functions where applicable.

The role structure separates the technical official roles, preparation roles, the competition director, and admin functions.

#### F.1.1.3 Protection of credentials, secrets and data in transit/at rest where required.

[RFP §7.1] Protection of credentials, secrets and data in transit/at rest where required.

All credentials or secrets are stored encrypted and transported in encrypted form.  They are never written to log files in clear. Account passwords use PBKDF2 encryption with HMAC-SHA256. 
Todo: tracker connection in database needs propping up.

#### F.1.1.4 Controlled remote support, removable media and patch/deployment processes.

[RFP §7.1] Controlled remote support, removable media and patch/deployment processes.

Security options are in the database or in external configuration files; updating to a fixed version is independent of the options. All future versions of OWLCMS will support the security requirements of IWF.

#### F.1.1.5 Tamper-resistant audit sufficient to reconstruct material actions.

[RFP §7.1] Tamper-resistant audit sufficient to reconstruct material actions.

The audit log is tamper-resistant and any tampering is detectable (the logger keeps its own copy of the lines and cryptographically signs blocks of lines).  The audit logs includes every single modification to the athlete and lifting data,and all events coming from devices (decisions etc.)

#### F.1.1.6 No public or anonymous path may write authoritative competition state.

[RFP §7.1] No public or anonymous path may write authoritative competition state.

The only writing to the database is done by authenticated users.  The database is the only source of written archivals, that are also cryptographically signed.

### F.1.2 Offline operation

[RFP §7.2] Core competition, officiating, timing, local displays and local official documents shall remain usable during loss of public internet or cloud connectivity.

Core competition, officiating, timing, local displays and local official documents shall remain usable during loss of public internet or cloud connectivity.

The normal mode of operation of OWLCMS is completely off-line, on a private local area network, with all key components wired (in particular the refereeing functions).

### F.1.3 Backup and recovery

#### F.1.3.1 Protected backup copies of data, configuration, rules, assets and audit.

[RFP §7.3] Protected backup copies of data, configuration, rules, assets and audit.

All files that constitute the OWLCMS state are in a single folder (with the exception of cryptographic key)  Any means of backup can be used – normally copying the folder to a removable device or off-site storage.

#### F.1.3.2 Timed restoration demonstrating the approved recovery objective.

[RFP §7.3] Timed restoration demonstrating the approved recovery objective.

Restoration of a saved state from scratch normally takes less than a minute.

#### F.1.3.3 Defined promotion, split-brain prevention, failback and reconciliation where redundancy is claimed.

[RFP §7.3] Defined promotion, split-brain prevention, failback and reconciliation where redundancy is claimed.

The system would not operate in tandem.  Checkpoint copies would be used.

#### F.1.3.4 Cold-start and rollback procedures using controlled recovery media.

[RFP §7.3] Cold-start and rollback procedures using controlled recovery media.

Checkpoint copies of the export format can be obtained on demand, and constitute full rollback points.  Restarting the system without a database file creates a minimal empty one that can be immediately loaded from a checkpoint.

### F.1.4 Manual continuity

[RFP §7.4] A manual Protocol and rehearsed manual-continuity procedure shall remain available. Staff shall know when to continue, when to STOP and how to reconcile manual records with the electronic state.

A manual Protocol and rehearsed manual-continuity procedure shall remain available. Staff shall know when to continue, when to STOP and how to reconcile manual records with the electronic state.

This is standard operational procedure.  A power outage, or a networking mishap can prevent use the system and cause a revert to a “manual cards and flags” mode.

OWLCMS has been tested in such situations before. As soon as the system is restored, someone can simply copy the information as it stands on the cards, even while lifting is going on that platform.  As soon as the information has been updated and matches the paper trail, the screens can be turned on for that platform.  All the lifting order and such is derived from the authoritative card data as entered.

Writing operational procedures as expected by IWF would take place before deployment.

### F.1.5 Monitoring

[RFP §7.5] Actionable monitoring shall cover critical services, devices, timing, storage, network and stale-state conditions, with clear escalation responsibilities.

Actionable monitoring shall cover critical services, devices, timing, storage, network and stale-state conditions, with clear escalation responsibilities.

The actual OTIS kit requires a router and, for each platform, a switch.  As part of the certified kit, this constitutes the monitoring boundary. The kit devices are attached directly to these and can be monitored (including remote logins).

# G. Integration

Interfaces, APIs/feeds, public/broadcast and Games/ORIS integrations.

## G.1 Interfaces

[RFP §6.3] Material interfaces shall document producer/consumer, purpose, version, schema/field mapping, identifiers, status/null rules, sequencing/revision, time source, latency, authentication, acknowledgement, retry/reconnect, monitoring and fallback.

Material interfaces shall document producer/consumer, purpose, version, schema/field mapping, identifiers, status/null rules, sequencing/revision, time source, latency, authentication, acknowledgement, retry/reconnect, monitoring and fallback.

### G.1.1 Device Protocol and Authentication

All the devices on the network communicate using MQTT messages to the OWLCMS server; see the specification on-line https://jflamy.github.io/owlcms4/#/100-AdvancedTopics/200-MQTTMessages

Standard MQTT authentication protocols are used (login name and password); crypted MQTT is available, but this is normally only used on cloud-based configurations, which is not the case for the proposed completely-local, closed-network configuration.

### G.1.2 Blue Owl USB-MQTT Interface

- Blue Owl devices use the same computing chip that is used on Arduino Uno or Nano, or on Arduino Mega

- The Blue Owl devices use a standard USB 2.0 interface to communicate with the computer that powers them.

- All communications with the computer are done serially on the same USB connection, using the publicly documented Firmata protocol.

- The Blue Owl program speaks Firmata to the devices and relays the information to OWLCMS

  - When a button is pressed on the device, this is immediately propagated by Blue Owl as an MQTT message to OWLCMS

  - Conversely, when OWLCMS issues a command to turn on an LED or buzzer, the Firmata message activates the corresponding ports.

  - Blue Owl itself will attempt reconnections every second in case of disconnection

  - The server detects and reports when a Blue Owl module disconnects and reconnects

  - Each Blue Owl interface keeps complete logs.

- The actual implementation of the software code used by the Blue Owl user interface to perform the interactions is publicly available at https://github.com/nemikor-solutions/blue-owl-core

- Schematics of the Blue Owl devices are included in Annex A.

### G.1.3 Direct MQTT Networking

Because the MQTT protocol is documented, any device that can be designed to listen to MQTT can just subscribe to the topics.  The VPT modules uses the MQTT messages to know when the clock is started and stopped, as an example. WiFi devices can also act as devices by talking directly to MQTT.

### G.1.4 Latency

Latency between the button press and reception of the MQTT message is under XX ms.

# H. Operations

Staffing, training, manuals, event support, escalation and incident response.

## H.1 Supplier responsibilities

### H.1.1 Provide accurate product and hardware baseline

[RFP §8.2] Provide accurate product and hardware baseline.

[To be completed]

### H.1.2 Provide complete technical submission and evidence package

[RFP §8.2] Provide complete technical submission and evidence package.

[To be completed]

### H.1.3 Make the declared system available for IWF testing and demonstrations

[RFP §8.2] Make the declared system available for IWF testing and demonstrations.

[To be completed]

### H.1.4 Correct deficiencies and complete regression testing when required

[RFP §8.2] Correct deficiencies and complete regression testing when required.

[To be completed]

### H.1.5 Notify IWF of material product, security, hardware, interface or rule-logic changes

[RFP §8.2] Notify IWF of material product, security, hardware, interface or rule-logic changes.

[To be completed]

### H.1.6 Maintain qualified support personnel and appropriate spares

[RFP §8.2] Maintain qualified support personnel and appropriate spares.

[To be completed]

### H.1.7 Protect IWF, athlete and event information and comply with applicable data-protection requirements.

[RFP §8.2] Protect IWF, athlete and event information and comply with applicable data-protection requirements.

[To be completed]

## H.2 Event deployment

### H.2.1 Only the approved baseline and event configuration may be used

[RFP §12.3] Only the approved baseline and event configuration may be used.

[To be completed]

### H.2.2 Actual hardware, software, network, power, displays, spares and staffing must match the approved event design.

[RFP §12.3] Actual hardware, software, network, power, displays, spares and staffing must match the approved event design.

[To be completed]

### H.2.3 Dress rehearsal and failure/recovery test shall be completed before readiness sign-off.

[RFP §12.3] Dress rehearsal and failure/recovery test shall be completed before readiness sign-off.

[To be completed]

### H.2.4 Substitutions require approved deviation or delta testing before use

[RFP §12.3] Substitutions require approved deviation or delta testing before use.

[To be completed]

## H.3 Support and incidents

### H.3.1 Defined escalation contacts and event support

[RFP §12.4] Defined escalation contacts and event support.

[To be completed]

### H.3.2 Critical incidents documented, contained, investigated and reported

[RFP §12.4] Critical incidents documented, contained, investigated and reported.

[To be completed]

### H.3.3 Serious incidents, unauthorised changes, false evidence, unsupported baselines or material security compromise may lead to restriction, suspension or withdrawal.

[RFP §12.4] Serious incidents, unauthorised changes, false evidence, unsupported baselines or material security compromise may lead to restriction, suspension or withdrawal.

[To be completed]

# I. Homologation

Test environment, evidence package, demonstration plan and FAT/SAT readiness.

## I.1 Evaluation and Homologation Process

### I.1.1 Administrative review

[RFP §10] Completeness, eligibility and declarations.

[To be completed]

### I.1.2 Technical review

[RFP §10] Architecture, functionality, hardware, interfaces, security and resilience.

[To be completed]

### I.1.3 Scope confirmation

[RFP §10] Freeze baseline, profile, capacity, dependencies and test plan.

[To be completed]

### I.1.4 FAT

[RFP §10] Controlled normal, edge, security, capacity and failure testing.

[To be completed]

### I.1.5 Corrective action

[RFP §10] Resolve deficiencies and provide regression evidence.

[To be completed]

### I.1.6 Operational validation

[RFP §10] Representative event tempo with operators, officials, displays, feeds and recovery.

[To be completed]

### I.1.7 Homologation decision

[RFP §10] Review requirements, hard gates, evidence and findings.

[To be completed]

### I.1.8 Supplier licence

[RFP §10] Complete legal and operational conditions.

[To be completed]

### I.1.9 Event SAT

[RFP §10] Check actual installation against approved baseline.

[To be completed]

### I.1.10 Surveillance

[RFP §10] Review incidents, changes, support and expiry.

[To be completed]

### I.1.11 Evidence

[RFP §10.1] Evidence shall be attributable to a specific baseline and test. Screenshots alone are insufficient where logs, physical inspection or live behaviour are necessary.

Evidence shall be attributable to a specific baseline and test. Screenshots alone are insufficient where logs, physical inspection or live behaviour are necessary.

Supplier response
[TO BE COMPLETED]

## I.2 Approval Criteria and Hard Gates

### I.2.1 HG-01 Sport rules & result correctness

[RFP §11] Incorrect sporting fact, rank, medal, record or rule-critical transition.

[To be completed]

### I.2.2 HG-02 Authoritative state & data integrity

[RFP §11] Split, duplicated, lost or silently overwritten accepted state.

[To be completed]

### I.2.3 HG-03 Timing & adjudication safety

[RFP §11] Unsafe or ambiguous timing, decision, Down signal or restart.

[To be completed]

### I.2.4 HG-04 Correction & audit

[RFP §11] Unauthorised, partial, untraceable or unreconciled correction.

[To be completed]

### I.2.5 HG-05 Continuity & recovery

[RFP §11] Uncontrolled loss of competition or unverified recovery.

[To be completed]

### I.2.6 HG-06 Security & least privilege

[RFP §11] Uncontrolled privilege, public write path or material compromise.

[To be completed]

### I.2.7 HG-07 Output parity & publication

[RFP §11] Contradictory outputs or untraceable final publication.

[To be completed]

### I.2.8 HG-08 Baseline reproducibility

[RFP §11] Unknown tested version, missing evidence or irreproducible installation.

[To be completed]

### I.2.9 HG-09 FOP readiness & safety

[RFP §11] Missing critical equipment, unsafe installation or unresolved obstruction.

[To be completed]

### I.2.10 Decision rules

#### I.2.10.1 All applicable mandatory requirements and tests must be disposed of

[RFP §11.1] All applicable mandatory requirements and tests must be disposed of.

[To be completed]

#### I.2.10.2 Applicable TCRR requirements and hard gates must pass

[RFP §11.1] Applicable TCRR requirements and hard gates must pass.

[To be completed]

#### I.2.10.3 No open Critical finding may remain

[RFP §11.1] No open Critical finding may remain.

[To be completed]

#### I.2.10.4 Traceability from requirement to evidence and test must be complete

[RFP §11.1] Traceability from requirement to evidence and test must be complete.

[To be completed]

#### I.2.10.5 Successful FAT, recovery and operational validation are required for the claimed profile.

[RFP §11.1] Successful FAT, recovery and operational validation are required for the claimed profile.

[To be completed]

#### I.2.10.6 Partial is not Pass

[RFP §11.1] Partial is not Pass.

[To be completed]

#### I.2.10.7 A profile limitation cannot change an IWF sport rule or bypass a hard gate

[RFP §11.1] A profile limitation cannot change an IWF sport rule or bypass a hard gate.

[To be completed]

# J. Commercial

Licence, implementation, equipment, support/maintenance, optional services and recurring charges.

## J.1 Commercial and Contractual Conditions

The Licensing Fee for the four-year period is US$20,000, payable on an annual basis of US$5,000 per year.

### J.1.1 IWF licensing fee

[RFP §13] The Licensing Fee for the four-year period is US$20,000, payable on an annual basis of US$5,000 per year.

US$5,000 per year / US$20,000 total

Assumptions: [TO BE COMPLETED]

### J.1.2 Licence

[RFP §13] Licence

[TO BE COMPLETED]

Assumptions: [TO BE COMPLETED]

### J.1.3 Implementation

[RFP §13] Implementation

[TO BE COMPLETED]

Assumptions: [TO BE COMPLETED]

### J.1.4 Equipment

[RFP §13] Equipment

[TO BE COMPLETED]

Assumptions: [TO BE COMPLETED]

### J.1.5 Support / maintenance

[RFP §13] Support / maintenance

[TO BE COMPLETED]

Assumptions: [TO BE COMPLETED]

### J.1.6 Optional services

[RFP §13] Optional services

[TO BE COMPLETED]

Assumptions: [TO BE COMPLETED]

### J.1.7 Recurring charges

[RFP §13] Recurring charges

[TO BE COMPLETED]

Assumptions: [TO BE COMPLETED]

# K. Legal

IP/licensing, warranties, liability, confidentiality, data protection, subcontractors and termination.

## K.1 Licence Conditions, Change Control and Event Deployment

### K.1.1 Licence validity

[RFP §12.1] The IWF licence shall be valid for a four-year period. The certificate shall state supplier, product/baseline, profiles, capacities, limitations, deviations, validity and surveillance conditions.

The IWF licence shall be valid for a four-year period. The final contract shall state the exact licence duration and renewal conditions.

Supplier response
[TO BE COMPLETED]

### K.1.2 Change control

#### K.1.2.1 Editorial / non-functional

[RFP §12.2] Controlled update and review.

[TO BE COMPLETED]

#### K.1.2.2 Minor cosmetic

[RFP §12.2] Supplier assessment and IWF notification where applicable.

[TO BE COMPLETED]

#### K.1.2.3 Significant - OS/database/firmware/interface/security/workflow/hardware

[RFP §12.2] Impact assessment and approved regression/delta test.

[TO BE COMPLETED]

#### K.1.2.4 Major - competition logic/timing/adjudication/records/state/failover

[RFP §12.2] Re-homologation before event use.

[TO BE COMPLETED]

#### K.1.2.5 Emergency security/stability correction

[RFP §12.2] Named approval, backup, rollback, targeted test and retrospective evidence.

[TO BE COMPLETED]

## K.2 Intellectual property

[RFP §12.5] IWF shall receive the rights, documentation, exports, interfaces and operational access necessary to use, verify, archive and maintain the approved system and competition records.

The supplier retains its intellectual property except where otherwise agreed. IWF shall receive the rights, documentation, exports, interfaces and operational access necessary to use, verify, archive and maintain the approved system and competition records. Final contractual wording will be agreed with the selected supplier.

Supplier response
[TO BE COMPLETED]

## K.3 Mandatory declarations

### K.3.1 Acceptance of the IWF TCRR as the controlling sporting rules

[RFP §9.1] Acceptance of the IWF TCRR as the controlling sporting rules.

[TO BE COMPLETED]

### K.3.2 Identification of all material limitations and dependencies

[RFP §9.1] Identification of all material limitations and dependencies.

[TO BE COMPLETED]

### K.3.3 Ability to reproduce the tested system from controlled installation material

[RFP §9.1] Ability to reproduce the tested system from controlled installation material.

[TO BE COMPLETED]

### K.3.4 Offline operation and tested recovery for the claimed profile

[RFP §9.1] Offline operation and tested recovery for the claimed profile.

[TO BE COMPLETED]

### K.3.5 No automatic licensing mechanism that can interrupt an active competition

[RFP §9.1] No automatic licensing mechanism that can interrupt an active competition.

[TO BE COMPLETED]

| Authorised signatory | Details |
| --- | --- |
| Name | [TO BE COMPLETED] |
| Title | [TO BE COMPLETED] |
| Signature | [TO BE COMPLETED] |
| Date | [TO BE COMPLETED] |

# L. Compliance matrix

Clear response to this tender and the applicable IWF assessment.

## RFP Section 2

| Requirement | Compliance | Reference | Justification |
| --- | --- | --- | --- |
| 2.1 Complete sport logic, mandatory FOP controls, screens and official documents. | Compliant | B.2.1 | Claimed: Yes. Fully compliant |
| 2.1-R02 M0 plus baseline control, audit, interfaces, offline operation, recovery, procedures, training, support and spares. | Compliant | B.2.2 | Claimed: Yes. The operation of both the software and hardware devices is fully documented. Spares for the devices are available for purchase. |
| 2.1-R03 M1 plus applicable VPT, public/live services, broadcast outputs, enhanced monitoring, high availability and full rehearsal. | Compliant | B.2.3 | Claimed: Yes. Video playback requires acquisition of additional video-capable equipment (laptops with video capture/gaming cards, cameras). OWLCMS has its own standalone VPT module that would be covered by this proposal. OWLCMS provides a set of TV Graphics for broadcasts, and its replay streams can be used as well. Supplier routinely does even-specific TV Graphics and has used the OWLCMS integration features to do so at large multisport games. |
| 2.1-R04 M2 plus approved Games/ORIS and programme-specific integrations. | Compliant | B.2.4 | Claimed: yes. Supplier has integrated OWLCMS with the ODF (Olympic Data Format) for Games integration in the past. Integration with specific games systems |
| 2.1-R05 Simultaneous platform operation with separation and capacity evidence. | Compliant | B.2.5 | Claimed: Yes. OWLCMS has supported up to 6 platforms simultaneously with extremely low computer and network usage. The program is verified using a stress test designed for this purpose. |
| 2.2 The licence certificate shall state the supplier and product. | Compliant | B.3.1 | [To be completed] |
| 2.2-R02 The licence certificate shall state the exact baseline. | Compliant | B.3.2 | Any stable version after version 69 meets the current requirements and is fully compliant with TCRR as published at the time of this tender. |
| 2.2-R03 The licence certificate shall state approved profiles and capacities. | Compliant | B.3.3 | The system has been tested to 5 concurrent platforms and 1600 participants, and can be stress-tested on demand. Evidence / reference: USAW Open 2024, USAW VWS Series 2025 and 2026, IMWA World Championships |
| 2.2-R04 The licence certificate shall state interfaces and critical dependencies. | Compliant | B.3.4 | The system has no critical dependencies. It can be run on any of Windows, Linux and macOS. All communications use the standard HTTP and MQTT protocols. The protocol by which all events and all data from the competition can be obtained to create additional interfaces is public. |
| 2.2-R05 The licence certificate shall state limitations and deviations. | Compliant | B.3.5 | None known. |
| 2.2-R06 The licence certificate shall state validity and surveillance conditions. | Compliant | B.3.6 | IWF does not acquire the right to issue licenses. IWF certifies that the given combination of products as proposed is compliant. IWF may acquire licenses from Supplier for its own use at competitions that are directly its authority. Organizers of multisport games, or of continental, regional and national and local competitions need to acquire their own licenses if they wish to use the IWF-certified version with the corresponding levels of support. Certification by IWF of a given version does not preclude the use same software or equipment where such certification is not needed or claimed. |

## RFP Section 3

| Requirement | Compliance | Reference | Justification |
| --- | --- | --- | --- |
| 3-R01 Import and validate athlete, federation/NOC, official, event, venue and record data; stable IDs and error reporting. | Compliant | E.1.1 | Data is converted to an Excel-style sheet with the athlete identifier, name, and all registration data. Competition information and dates are stored in the program database (all information is stored in a relational database). The initial schedule is part of the data intake, and an initial assignment of athletes to sessions is done. The input format handles multiple age groups and supports team allocation. The intake format can be exported, edited off-line, and re-ingested without loss of information. The same applies to schedule changes and referee assignments that can be modified in bulk until stabilized. |
| 3-R02 Corrections, approvals, random lots, grouping, timetable and Technical Official assignments. | Compliant | E.1.2 | The program prints VFE verification sheets that show exactly which athlete is in which category or categories. There are also team membership teams to verify that all team allocations are correct. These sheets can be signed by team representatives to facilitate the VFE process and reduce the number of errors, or used by the TIS team to do the capture and then get them signed. |
| 3-R03 Approved Start List and controlled distribution/versioning. | Compliant | E.1.3 | OWLCMS as proposed is able to produce a fully compliant start book. Preliminary versions can be published to list participants, with subsequent editions adding the schedule and the officials assignments. Once converted to the OWLCMS format, any number of records can be handled. Field-of-play formats are also available (actual session assignments and schedule) for direct use by coaches and technical officials. |
| 3-R04 Correct timing, bodyweight, first attempts, 20 kg checks, start numbers, signatures and closed snapshot. | Compliant | E.1.4 | OWLCMS produces field-tested Weigh-in forms to capture both the weight-on-scale and deducted weight. All applicable rule are checked, including 20 kg and qualifying totals where applicable. The form captures all the required signature. Compliant athlete cards are likewise printed. A special data entry mode is used for quick capture of weigh-in data. A post-weigh-in snapshot of the database is taken. Start numbers are cross-checked at the end of the weigh-in and checked against allocated bib numbers. A speaker introduction sheet for athletes and officials is produced, as are the empty protocols and jury member sheets. |
| 3-R05 Calling order, declarations/changes, 60/120-second timing, referee decisions, records and live outputs. | Compliant | E.1.5 | Full compliance with the competition flow is attained. All marshal data entry is immediately handled by the system and all lifting order or requested weights are instantly updated on all displays and scoreboards. All the normal timing decisions are automatic (the clock is automatically set to the right allocated time for each athlete, including on fast sequences of changes). Only explicit loading incidents require a reset. All attempts are checked against the 20 kg rule and the required changes are explicitly visible to the marshal. Incomplete situations that do not fully restore the 20kg rule are explicitly visible. |
| 3-R06 Controlled Snatch-to-Clean & Jerk transition and synchronized displays. | Compliant | E.1.6 | Default rules for longer breaks (few athletes) and shorter breaks (large sessions) are both available. The session schedule also allows changing the break duration. This can also be done after the session has started (under the assumption the meet director informs all concerned). Transition from the snatch to the clean & jerk break at the end of the snatch is automatic. A jury interruption or challenge on the last requires the time keeper starts the break once the situation has been resolved. |
| 3-R07 Calling order, weight changes, 20 kg enforcement, totals, ranks and records. | Compliant | E.1.7 | The same completely automatic timing and weight change flow as in the snatch takes place. |
| 3-R08 STOP, Jury, challenge/VPT where applicable, additional attempts, correction, withdrawal and restart. | Compliant | E.1.8 | The jury can start a deliberation or accept a challenge using a physical jury keypad button or using a jury tablet. A flashing STOP sign is shown to the athlete on the platform, and all attempt boards and displays indicate the interruption. Transmission of the jury reversal code to the speaker can be done the traditional way (by the technical controller), or automatically when the jury president chooses the reversal code. In either case, the jury or challenge decision is shown as the speaker announces it. The system can optionally display jury decisions on the scoreboards and the attempt boards. This capability is optional because it is not formally described in TCRR. |
| 3-R09 Protocol reconciliation, signatures/status, results, classifications and records. | Compliant | E.1.9 | A session protocol sheet is available immediately after the session that contains all the requisite information and each occurrence of an improved record. This is normally signed by the jury president. After a session, any correction required can be made via the system to ensure an audit trail to match whatever decisions were written on the protocols. |
| 3-R10 Medallists, public/broadcast outputs, communications and revised outputs. | Compliant | E.1.10 | The system distinguishes two types of scoreboards. Public scoreboards are normally styled to match the event colors, and switch content during ceremonies. OWLCMS will automatically switch during introductions. During medal ceremonies, the technical table (or the speaker themselves) can change the display to show the medals. In multi-age or multi-federation championships, this allows correct control of what is shown to the public. If broadcasters elect to use the video layout screens as provided by OWLCMS, the changes are also automatic. |
| 3-R11 Final dataset, official outputs, logs, revisions, evidence and machine-readable export. | Compliant | E.1.11 | The system distinguishes two types of scoreboards. Public scoreboards are normally styled to match the event colors, and switch content during ceremonies. OWLCMS will automatically switch during introductions. During medal ceremonies, the technical table (or the speaker themselves) can change the display to show the medals. In multi-age or multi-federation championships, this allows correct control of what is shown to the public. If broadcasters elect to use the video layout screens as provided by OWLCMS, the changes are also automatic. |
| 3.1-R01 Snatch followed by Clean & Jerk, maximum three attempts per lift, correct Total calculation. | Compliant | E.1.12.1 | Fully compliant with TCRR Evidence / reference: Explicitly in database schema, automated tests. |
| 3.1-R02 Whole-kilogram weights and applicable minimum starting weights. | Compliant | E.1.12.2 | Fully compliant with TCRR Evidence / reference: Explicitly in database schema, automated tests. |
| 3.1-R03 Correct calling order, including weight, attempt number, previous-attempt sequence and start number. | Compliant | E.1.12.3 | Fully compliant with TCRR Evidence / reference: Automated tests for all the change sequences. |
| 3.1-R04 Permitted weight changes, final-call and consecutive-attempt rules. | Compliant | E.1.12.4 | Fully compliant with TCRR Evidence / reference: Automated tests for all the change sequences and for the two-minute situations. Systematic recalculation of lift order to detect illegal moving down, with automatic tests. |
| 3.1-R05 Withdrawals, no-lift/no-Total conditions, disqualification and additional attempts. | Compliant | E.1.12.5 | Fully compliant with TCRR Evidence / reference: Automated tests of the total, disqualification and withdrawals. |
| 3.1-R06 Material corrections recalculate affected rankings, records and outputs. | Compliant | E.1.12.6 | Fully compliant with TCRR Evidence / reference: Any change to the lifting data causes a recalculation of all affected items. Automated tests for the ranking and records calculations. |

## RFP Section 4

| Requirement | Compliance | Reference | Justification |
| --- | --- | --- | --- |
| 4.1-R01 Authoritative electronic countdown in one-second increments, configurable up to at least 15 minutes. | Compliant | E.2.1.1 | Fully compliant with TCRR Evidence / reference: Timekeeper controls all break durations, no limit on duration. |
| 4.1-R02 Automatic 60- and 120-second attempt clocks and applicable introduction/break countdowns. | Compliant | E.2.1.2 | Fully compliant with TCRR Evidence / reference: Clock ownership is automatic, no intervention required to set the clock to 1:00, 2:00 or to the time remaining for the athlete. Transition between countdowns (introduction to snatch for example) is automatic. Can be overridden as required. |
| 4.1-R03 Correct start, stop, restart and STOP behaviour according to the rules. | Compliant | E.2.1.3 | Fully compliant with TCRR Evidence / reference: As per TCRR |
| 4.1-R04 All connected displays use the same authoritative time state within the approved tolerance. | Compliant | E.2.1.4 | Fully compliant with TCRR Evidence / reference: Time is kept on the OWLCMS server; when sent to remote devices, actual universal time reference for timer termination so the device can compensate for eventual small clock drift. All devices are time-synchronized. |
| 4.1-R05 Time corrections are authorised, recorded and auditable. | Compliant | E.2.1.5 | Fully compliant with TCRR Evidence / reference: The displays are NTP-synchronized at the operating system level by default. Verifying time synchronization is part of the technical setup. |
| 4.2-R01 Three independent referee control boxes with white/red decisions and required indicators. | Compliant | E.2.2.1 | Fully compliant with TCRR Evidence / reference: Nemikor referee buttons have visual indicator, sound, and vibration. |
| 4.2-R02 Correct majority/Down signal, reminder and reversal-window behaviour. | Compliant | E.2.2.2 | Fully compliant with TCRR Evidence / reference: Implemented by software. Reversal rules are as per TCRR. Manual reversal of a decision following flags can easily be done by the technical table. |
| 4.2-R03 Jury visibility of individual referee decisions before public reveal. | Compliant | E.2.2.3 | Fully compliant with TCRR Evidence / reference: Live referee decisions and reversal are shown to the jury both on the keypad and on the jury console screen or tablet display if used. |
| 4.2-R04 Support for approved three- and five-member Jury configurations. | Compliant | E.2.2.4 | Fully compliant with TCRR Evidence / reference: Jury size is selectable, |
| 4.2-R05 Individual Jury decision controls and protected independence between members. | Compliant | E.2.2.5 | Fully compliant with TCRR Evidence / reference: Individual jury buttons are provided as per TCRR. Decision protection housings are not a TCRR requirement but will be addressed. |
| 4.2-R06 Material decisions and actions are attributable and time-stamped. | Compliant | E.2.2.6 | Fully compliant with TCRR Evidence / reference: Full audit log of every single decision, timestamped to the millisecond, with the originating station or device. |
| 4.3-R01 At minimum, the TIS shall provide System health. | Compliant | E.2.3.1 | A technical log is always available for detailed technical operations. The memory, disk and networking usage are extremely low, the system gauges confirm this (the system never exceeds 20%, and is usually well below 10, even for extremely large meets). |
| 4.3-R02 At minimum, the TIS shall provide Event setup. | Compliant | E.2.3.2 | All competition information setup is done using interactive screens. Green is used in the software to indicate common actions. |
| 4.3-R04 At minimum, the TIS shall provide lots/groups/timetable. | Compliant | E.2.3.2.2 | Normally an import from a competitions registration system is used to set up the initial schedule and assign athletes assigned to A/B/C… groups. At the scale of IWF events, this is most likely done using output of the registration system in spreadsheet format. This is then converted to fit the documented OWLCMS format (this has now become trivial with human assistance tools). All data exchange formats for OWLCMS are public. Once this is done all athletes can be edited in the program, and late entries added by hand if needed. The registration screen allows modification of the allocated session, moving the athlete to another group, adjusting eligibility to multiple age groups as appropriate. The other tabs on the registration form cover things like record eligibility (in joint jurisdiction championships, not all athletes are eligible to improve all records) The session timetable is editable. |
| 4.3-R07 At minimum, the TIS shall provide Weigh-in. | Compliant | E.2.3.4 | The weigh-in data entry form is intentionally the same format as the registration form. All relevant checks for the 20 kg rule are applied at data entry, in case they were missed by the weigh-in Tos. After weigh-in, the page allows the creation of all the relevant forms for the speaker and for the jury. |
| 4.3-R08 At minimum, the TIS shall provide Chief Marshal. | Compliant | E.2.3.5 | The marshal table has a station that shows all athletes (in start order or lifting order as preferred by the TO operating). Clicking on an athlete brings up a card that is laid out exactly as the athlete card, and automatically brings focus to the right cell, such that typing and hitting Enter can be used for fast data entry. 20kg rule violations and late changes are immediately made visible |
| 4.3-R09 At minimum, the TIS shall provide Timekeeper. | Compliant | E.2.3.6 | The timekeeper can control the breaks (in addition to the main technical table or the speaker having the same feature depending on the skill and comfort levels). The delay can be set to a clock or duration as relevant for the type of break. During the competition a tablet-friendly display is used, that can be used together with the Blue Owl buttons, as preferred. |
| 4.3-R11 At minimum, the TIS shall provide Competition supervisor. | Compliant | E.2.3.8 | The competition supervisor screen is the as the speaker, with the addition of the decision lights that can be seen live as an option. This is because the switches between sessions, and breaks, and lift overrides after a late referee flag call can all be done from that screen. Depending on competitions, the speaker screen will be made “passive” if the technical table assumes those tasks instead of an ITO1 comfortable with technology. In addition, OWLCMS is launched from a control panel that gives additional options such as viewing the technical log going live. The competition supervisor can watch the competition-level audit log as it is written. |
| 4.3-R12 At minimum, the TIS shall provide Referee monitoring. | Compliant | E.2.3.9 | The competition director screen can see the live referee lights, as does the jury. The audit log shows each individual decision, and each weight change from the marshals. |
| 4.3-R13 At minimum, the TIS shall provide Jury control. | Compliant | E.2.3.10 | The jury control can be done entirely using a Blue Owl jury control keypad In the context of an IWF meet, it becomes desirable to accelerate the transfer of decisions from the jury to the speaker. To this effect, a tablet-style jury display is available that has the same features as the keypad Use of this tablet-style enables, in case of a “no lift” decision, the reason to be picked directly. When this is used, the Speaker and Competition Supervisor screens get the reason immediately, and can resume the competition after announcing it. |
| 4.3-R14 At minimum, the TIS shall provide Attempt boards. | Compliant | E.2.3.11 | Attempt boards show all the information required by TCRR. They can also display the flag of the country. If one or more record attempts take place the attempt boards show it as a rolling banner. |
| 4.3-R15 At minimum, the TIS shall provide Scoreboards. | Compliant | E.2.3.12 | Scoreboards contain all the requisite information. In addition, the top line contains the current attempt. The bottom line contains additional information, such as the various championships, rank if successful, jury decisions etc.). This is the warmup room layout, that does not change during ceremonies so coaches can count attempts. Scoreboard during session with previous leaders In the main room, styling will be applied as required by the event, and the top row is used as a banner. A row at the bottom carries will carry the clock and other attempt information. Main Room Scoreboard with previous leaders Full-fledged video production companies can replace this scoreboard and the TV Graphics by using the extension capabilities provided by OWLCMS. The extension protocol is documented with a reference implementation at https://github.com/owlcms/tracker-core . |
| 4.3-R16 At minimum, the TIS shall provide RTDS/warm-up. | Compliant | E.2.3.13 | In OWLCMS all the screens are RTDS (Real-time Display System) When decisions are given, they are shown both on the attempt board and on the scoreboards. If jury decisions are being shown, these appear as well. The bottom line then shows the athlete under review, whereas the top line shows the athlete getting ready (whose clock is not running). Jury Decision for current athlete |
| 4.3-R17 At minimum, the TIS shall provide STOP. | Compliant | E.2.3.14 | During deliberation, challenge, technical issue or other stoppage, the reason is shown Flashing Stop |
| 4.3-R18 At minimum, the TIS shall provide Records. | Compliant | E.2.3.15 | Record Attempt Attempt Board Record Attempt Scoreboard |
| 4.3-R19 At minimum, the TIS shall provide Results. | Compliant | E.2.3.16 | End of session rankings with medals |
| 4.3-R20 At minimum, the TIS shall provide Other approved states. | Compliant | E.2.3.17 | [TO BE COMPLETED] |
| 4.4-R01 At minimum, the TIS shall provide Start Lists. | Compliant | E.2.4.1 | [TO BE COMPLETED] |
| 4.4-R02 At minimum, the TIS shall provide Start List Package. | Compliant | E.2.4.2 | [TO BE COMPLETED] |
| 4.4-R03 At minimum, the TIS shall provide Weigh-in List. | Compliant | E.2.4.3 | [TO BE COMPLETED] |
| 4.4-R04 At minimum, the TIS shall provide Athlete's Cards. | Compliant | E.2.4.4 | [TO BE COMPLETED] |
| 4.4-R05 At minimum, the TIS shall provide Empty Protocol. | Compliant | E.2.4.5 | [TO BE COMPLETED] |
| 4.4-R06 At minimum, the TIS shall provide Manual Protocol. | Compliant | E.2.4.6 | [TO BE COMPLETED] |
| 4.4-R07 At minimum, the TIS shall provide Protocol. | Compliant | E.2.4.7 | [TO BE COMPLETED] |
| 4.4-R08 At minimum, the TIS shall provide Results. | Compliant | E.2.4.8 | [TO BE COMPLETED] |
| 4.4-R09 At minimum, the TIS shall provide Team Classification/Placing. | Compliant | E.2.4.9 | [TO BE COMPLETED] |
| 4.4-R10 At minimum, the TIS shall provide Records. | Compliant | E.2.4.10 | [TO BE COMPLETED] |
| 4.4-R11 At minimum, the TIS shall provide Final Results Package. | Compliant | E.2.4.11 | [TO BE COMPLETED] |
| 4.4-R12 At minimum, the TIS shall provide Results Book where applicable. | Compliant | E.2.4.12 | [TO BE COMPLETED] |
| 4.4-R13 At minimum, the TIS shall provide Machine-readable export. | Compliant | E.2.4.13 | [TO BE COMPLETED] |
| 4.4-R14 At minimum, the TIS shall provide Distribution records. | Compliant | E.2.4.14 | [TO BE COMPLETED] |
| 4.4-R15 At minimum, the TIS shall provide Final evidence/archive. | Compliant | E.2.4.15 | Paper, PDF and machine-readable outputs are delivery channels; none may become an undocumented alternative source of sporting truth. Supplier response [TO BE COMPLETED] |

## RFP Section 5

| Requirement | Compliance | Reference | Justification |
| --- | --- | --- | --- |
| 5-R01 Controlled capacity for authoritative state and approved services, with spare/recovery path. | Compliant | D.1.1 | [To be completed] |
| 5-R02 Dedicated, role-controlled competition administration and operation positions. | Compliant | D.1.2 | [To be completed] |
| 5-R03 Protected authoritative storage, backup and tested recovery. | Compliant | D.1.3 | [To be completed] |
| 5-R04 Three independent red/white referee control devices. | Compliant | D.1.4 | [To be completed] |
| 5-R05 Individual devices for applicable Jury members plus President controls. | Compliant | D.1.5 | [To be completed] |
| 5-R06 Authoritative timer/controller with approved displays and audio. | Compliant | D.1.6 | [To be completed] |
| 5-R07 Competition and warm-up presentation with mandatory athlete/attempt/weight/clock/decision data. | Compliant | D.1.7 | [To be completed] |
| 5-R08 Real-time competition and warm-up displays with correct ranking and state. | Compliant | D.1.8 | [To be completed] |
| 5-R09 Approved cameras, recording/replay and Jury output for events requiring VPT. | Compliant | D.1.9 | [To be completed] |
| 5-R10 Documented, segmented, monitored event network with common time. | Compliant | D.1.10 | [To be completed] |
| 5-R11 Controlled read-only path from accepted state to approved consumers. | Compliant | D.1.11 | [To be completed] |
| 5-R12 Reliable official documents with fallback. | Compliant | D.1.12 | [To be completed] |
| 5-R13 Protection and approved autonomy for critical equipment. | Compliant | D.1.13 | [To be completed] |
| 5-R14 Health, service, device, storage, time and stale-state monitoring. | Compliant | D.1.14 | [To be completed] |
| 5-R15 Rapidly deployable replacements for critical equipment families. | Compliant | D.1.15 | [To be completed] |
| 5-R16 Safe, labelled cabling and equipment placement without obstruction or sightline conflicts. | Compliant | D.1.16 | [To be completed] |
| 5.1 For each licensed event, the supplier shall maintain make/model, quantity, serial or asset ID, firmware, location, connections, power source, owner, health status, spare path and approved deviations. | Compliant | D.1.17 | For each licensed event, the supplier shall maintain make/model, quantity, serial or asset ID, firmware, location, connections, power source, owner, health status, spare path and approved deviations. Supplier response [TO BE COMPLETED] |

## RFP Section 6

| Requirement | Compliance | Reference | Justification |
| --- | --- | --- | --- |
| 6.1-R01 Implement the effective IWF rule pack and support controlled updates. | Compliant | C.1.1.1 | [To be completed] |
| 6.1-R02 Competition-critical functions shall not depend on unavailable internet, licence-server or payment services. | Compliant | C.1.1.2 | [To be completed] |
| 6.1-R03 Exact production version, configuration, database schema, rule pack and relevant assets shall be identifiable and reproducible. | Compliant | C.1.1.3 | [To be completed] |
| 6.1-R04 Installation and rollback shall be documented and repeatable. | Compliant | C.1.1.4 | [To be completed] |
| 6.1-R05 Automatic updates shall not occur during an event freeze unless approved. | Compliant | C.1.1.5 | [To be completed] |
| 6.2-R01 Every sporting fact shall have one declared authoritative owner. | Compliant | C.1.2.1 | [To be completed] |
| 6.2-R02 Derived screens, feeds, documents and caches shall be traceable to the accepted state. | Compliant | C.1.2.2 | [To be completed] |
| 6.2-R03 Corrections shall be authorised, atomic, attributable and propagated. | Compliant | C.1.2.3 | [To be completed] |
| 6.2-R04 Duplicate, lost, conflicting or out-of-order transactions shall be prevented or detected. | Compliant | C.1.2.4 | [To be completed] |
| 6.2-R05 A complete machine-readable event export shall be available to IWF. | Compliant | C.1.2.5 | [To be completed] |
| 6.3 Material interfaces shall document producer/consumer, purpose, version, schema/field mapping, identifiers, status/null rules, sequencing/revision, time source, latency, authentication, acknowledgement, retry/reconnect, monitoring and fallback. | Compliant | G.1 | Material interfaces shall document producer/consumer, purpose, version, schema/field mapping, identifiers, status/null rules, sequencing/revision, time source, latency, authentication, acknowledgement, retry/reconnect, monitoring and fallback. |
| 6.4 All third-party software, libraries, services, licences and hardware necessary for operation shall be declared, including version, ownership, support status and cloud/public-internet dependency. | Compliant | C.2 | All third-party software, libraries, services, licences and hardware necessary for operation shall be declared, including version, ownership, support status and cloud/public-internet dependency. All dependencies used in OWLCMS and Blue Owl are public and available on public servers. |

## RFP Section 7

| Requirement | Compliance | Reference | Justification |
| --- | --- | --- | --- |
| 7.1-R01 Least-privilege access by role and device identity. | Compliant | F.1.1.1 | OWLCMS supports separate logins for every role, and where relevant, separate logins for each role and platform. Devices are identified per platform and authenticated using the MQTT identity of the MQTT interface. |
| 7.1-R02 Separation of competition, device, public/display, database and management functions where applicable. | Compliant | F.1.1.2 | The role structure separates the technical official roles, preparation roles, the competition director, and admin functions. |
| 7.1-R03 Protection of credentials, secrets and data in transit/at rest where required. | Compliant | F.1.1.3 | All credentials or secrets are stored encrypted and transported in encrypted form. They are never written to log files in clear. Account passwords use PBKDF2 encryption with HMAC-SHA256. Todo: tracker connection in database needs propping up. |
| 7.1-R04 Controlled remote support, removable media and patch/deployment processes. | Compliant | F.1.1.4 | Security options are in the database or in external configuration files; updating to a fixed version is independent of the options. All future versions of OWLCMS will support the security requirements of IWF. |
| 7.1-R05 Tamper-resistant audit sufficient to reconstruct material actions. | Compliant | F.1.1.5 | The audit log is tamper-resistant and any tampering is detectable (the logger keeps its own copy of the lines and cryptographically signs blocks of lines). The audit logs includes every single modification to the athlete and lifting data,and all events coming from devices (decisions etc.) |
| 7.1-R06 No public or anonymous path may write authoritative competition state. | Compliant | F.1.1.6 | The only writing to the database is done by authenticated users. The database is the only source of written archivals, that are also cryptographically signed. |
| 7.2 Core competition, officiating, timing, local displays and local official documents shall remain usable during loss of public internet or cloud connectivity. | Compliant | F.1.2 | Core competition, officiating, timing, local displays and local official documents shall remain usable during loss of public internet or cloud connectivity. The normal mode of operation of OWLCMS is completely off-line, on a private local area network, with all key components wired (in particular the refereeing functions). |
| 7.3-R01 Protected backup copies of data, configuration, rules, assets and audit. | Compliant | F.1.3.1 | All files that constitute the OWLCMS state are in a single folder (with the exception of cryptographic key) Any means of backup can be used – normally copying the folder to a removable device or off-site storage. |
| 7.3-R02 Timed restoration demonstrating the approved recovery objective. | Compliant | F.1.3.2 | Restoration of a saved state from scratch normally takes less than a minute. |
| 7.3-R03 Defined promotion, split-brain prevention, failback and reconciliation where redundancy is claimed. | Compliant | F.1.3.3 | The system would not operate in tandem. Checkpoint copies would be used. |
| 7.3-R04 Cold-start and rollback procedures using controlled recovery media. | Compliant | F.1.3.4 | Checkpoint copies of the export format can be obtained on demand, and constitute full rollback points. Restarting the system without a database file creates a minimal empty one that can be immediately loaded from a checkpoint. |
| 7.4 A manual Protocol and rehearsed manual-continuity procedure shall remain available. Staff shall know when to continue, when to STOP and how to reconcile manual records with the electronic state. | Compliant | F.1.4 | A manual Protocol and rehearsed manual-continuity procedure shall remain available. Staff shall know when to continue, when to STOP and how to reconcile manual records with the electronic state. This is standard operational procedure. A power outage, or a networking mishap can prevent use the system and cause a revert to a “manual cards and flags” mode. OWLCMS has been tested in such situations before. As soon as the system is restored, someone can simply copy the information as it stands on the cards, even while lifting is going on that platform. As soon as the information has been updated and matches the paper trail, the screens can be turned on for that platform. All the lifting order and such is derived from the authoritative card data as entered. Writing operational procedures as expected by IWF would take place before deployment. |
| 7.5 Actionable monitoring shall cover critical services, devices, timing, storage, network and stale-state conditions, with clear escalation responsibilities. | Compliant | F.1.5 | Actionable monitoring shall cover critical services, devices, timing, storage, network and stale-state conditions, with clear escalation responsibilities. The actual OTIS kit requires a router and, for each platform, a switch. As part of the certified kit, this constitutes the monitoring boundary. The kit devices are attached directly to these and can be monitored (including remote logins). |

## RFP Section 8

| Requirement | Compliance | Reference | Justification |
| --- | --- | --- | --- |
| 8.1-R01 Legally established entity able to contract with IWF. | Compliant | A.1.1.1 | [To be completed] |
| 8.1-R02 Relevant experience delivering competition scoring/results or comparable mission-critical systems. | Compliant | A.1.1.2 | [To be completed] |
| 8.1-R03 Ability to accept end-to-end responsibility for the complete system and declared third-party components. | Compliant | A.1.1.3 | [To be completed] |
| 8.1-R04 Qualified technical, competition-operations and support personnel. | Compliant | A.1.1.4 | [To be completed] |
| 8.1-R05 International training, event support and incident-response capability. | Compliant | A.1.1.5 | [To be completed] |
| 8.1-R06 Ability to maintain the approved baseline for the licence period. | Compliant | A.1.1.6 | [To be completed] |
| 8.2-R01 Provide accurate product and hardware baseline. | Compliant | H.1.1 | [To be completed] |
| 8.2-R02 Provide complete technical submission and evidence package. | Compliant | H.1.2 | [To be completed] |
| 8.2-R03 Make the declared system available for IWF testing and demonstrations. | Compliant | H.1.3 | [To be completed] |
| 8.2-R04 Correct deficiencies and complete regression testing when required. | Compliant | H.1.4 | [To be completed] |
| 8.2-R05 Notify IWF of material product, security, hardware, interface or rule-logic changes. | Compliant | H.1.5 | [To be completed] |
| 8.2-R06 Maintain qualified support personnel and appropriate spares. | Compliant | H.1.6 | [To be completed] |
| 8.2-R07 Protect IWF, athlete and event information and comply with applicable data-protection requirements. | Compliant | H.1.7 | [To be completed] |

## RFP Section 9

| Requirement | Compliance | Reference | Justification |
| --- | --- | --- | --- |
| 9.1-R01 Acceptance of the IWF TCRR as the controlling sporting rules. | Compliant | K.3.1 | [TO BE COMPLETED] |
| 9.1-R02 Identification of all material limitations and dependencies. | Compliant | K.3.2 | [TO BE COMPLETED] |
| 9.1-R03 Ability to reproduce the tested system from controlled installation material. | Compliant | K.3.3 | [TO BE COMPLETED] |
| 9.1-R04 Offline operation and tested recovery for the claimed profile. | Compliant | K.3.4 | [TO BE COMPLETED] |
| 9.1-R05 No automatic licensing mechanism that can interrupt an active competition. | Compliant | K.3.5 | [TO BE COMPLETED] |

## RFP Section 10

| Requirement | Compliance | Reference | Justification |
| --- | --- | --- | --- |
| 10-S01 Completeness, eligibility and declarations. | Compliant | I.1.1 | [To be completed] |
| 10-S02 Architecture, functionality, hardware, interfaces, security and resilience. | Compliant | I.1.2 | [To be completed] |
| 10-S03 Freeze baseline, profile, capacity, dependencies and test plan. | Compliant | I.1.3 | [To be completed] |
| 10-S04 Controlled normal, edge, security, capacity and failure testing. | Compliant | I.1.4 | [To be completed] |
| 10-S05 Resolve deficiencies and provide regression evidence. | Compliant | I.1.5 | [To be completed] |
| 10-S06 Representative event tempo with operators, officials, displays, feeds and recovery. | Compliant | I.1.6 | [To be completed] |
| 10-S07 Review requirements, hard gates, evidence and findings. | Compliant | I.1.7 | [To be completed] |
| 10-S08 Complete legal and operational conditions. | Compliant | I.1.8 | [To be completed] |
| 10-S09 Check actual installation against approved baseline. | Compliant | I.1.9 | [To be completed] |
| 10-S10 Review incidents, changes, support and expiry. | Compliant | I.1.10 | [To be completed] |
| 10.1 Evidence shall be attributable to a specific baseline and test. Screenshots alone are insufficient where logs, physical inspection or live behaviour are necessary. | Compliant | I.1.11 | Evidence shall be attributable to a specific baseline and test. Screenshots alone are insufficient where logs, physical inspection or live behaviour are necessary. Supplier response [TO BE COMPLETED] |

## RFP Section 11

| Requirement | Compliance | Reference | Justification |
| --- | --- | --- | --- |
| HG-01 Incorrect sporting fact, rank, medal, record or rule-critical transition. | Compliant | I.2.1 | [To be completed] |
| HG-02 Split, duplicated, lost or silently overwritten accepted state. | Compliant | I.2.2 | [To be completed] |
| HG-03 Unsafe or ambiguous timing, decision, Down signal or restart. | Compliant | I.2.3 | [To be completed] |
| HG-04 Unauthorised, partial, untraceable or unreconciled correction. | Compliant | I.2.4 | [To be completed] |
| HG-05 Uncontrolled loss of competition or unverified recovery. | Compliant | I.2.5 | [To be completed] |
| HG-06 Uncontrolled privilege, public write path or material compromise. | Compliant | I.2.6 | [To be completed] |
| HG-07 Contradictory outputs or untraceable final publication. | Compliant | I.2.7 | [To be completed] |
| HG-08 Unknown tested version, missing evidence or irreproducible installation. | Compliant | I.2.8 | [To be completed] |
| HG-09 Missing critical equipment, unsafe installation or unresolved obstruction. | Compliant | I.2.9 | [To be completed] |
| 11.1-R01 All applicable mandatory requirements and tests must be disposed of. | Compliant | I.2.10.1 | [To be completed] |
| 11.1-R02 Applicable TCRR requirements and hard gates must pass. | Compliant | I.2.10.2 | [To be completed] |
| 11.1-R03 No open Critical finding may remain. | Compliant | I.2.10.3 | [To be completed] |
| 11.1-R04 Traceability from requirement to evidence and test must be complete. | Compliant | I.2.10.4 | [To be completed] |
| 11.1-R05 Successful FAT, recovery and operational validation are required for the claimed profile. | Compliant | I.2.10.5 | [To be completed] |
| 11.1-R06 Partial is not Pass. | Compliant | I.2.10.6 | [To be completed] |
| 11.1-R07 A profile limitation cannot change an IWF sport rule or bypass a hard gate. | Compliant | I.2.10.7 | [To be completed] |

## RFP Section 12

| Requirement | Compliance | Reference | Justification |
| --- | --- | --- | --- |
| 12.1 The IWF licence shall be valid for a four-year period. The certificate shall state supplier, product/baseline, profiles, capacities, limitations, deviations, validity and surveillance conditions. | Compliant | K.1.1 | The IWF licence shall be valid for a four-year period. The final contract shall state the exact licence duration and renewal conditions. Supplier response [TO BE COMPLETED] |
| 12.2-R01 Controlled update and review. | Compliant | K.1.2.1 | [TO BE COMPLETED] |
| 12.2-R02 Supplier assessment and IWF notification where applicable. | Compliant | K.1.2.2 | [TO BE COMPLETED] |
| 12.2-R03 Impact assessment and approved regression/delta test. | Compliant | K.1.2.3 | [TO BE COMPLETED] |
| 12.2-R04 Re-homologation before event use. | Compliant | K.1.2.4 | [TO BE COMPLETED] |
| 12.2-R05 Named approval, backup, rollback, targeted test and retrospective evidence. | Compliant | K.1.2.5 | [TO BE COMPLETED] |
| 12.3-R01 Only the approved baseline and event configuration may be used. | Compliant | H.2.1 | [To be completed] |
| 12.3-R02 Actual hardware, software, network, power, displays, spares and staffing must match the approved event design. | Compliant | H.2.2 | [To be completed] |
| 12.3-R03 Dress rehearsal and failure/recovery test shall be completed before readiness sign-off. | Compliant | H.2.3 | [To be completed] |
| 12.3-R04 Substitutions require approved deviation or delta testing before use. | Compliant | H.2.4 | [To be completed] |
| 12.4-R01 Defined escalation contacts and event support. | Compliant | H.3.1 | [To be completed] |
| 12.4-R02 Critical incidents documented, contained, investigated and reported. | Compliant | H.3.2 | [To be completed] |
| 12.4-R03 Serious incidents, unauthorised changes, false evidence, unsupported baselines or material security compromise may lead to restriction, suspension or withdrawal. | Compliant | H.3.3 | [To be completed] |
| 12.5 IWF shall receive the rights, documentation, exports, interfaces and operational access necessary to use, verify, archive and maintain the approved system and competition records. | Compliant | K.2 | The supplier retains its intellectual property except where otherwise agreed. IWF shall receive the rights, documentation, exports, interfaces and operational access necessary to use, verify, archive and maintain the approved system and competition records. Final contractual wording will be agreed with the selected supplier. Supplier response [TO BE COMPLETED] |

## RFP Section 13

| Requirement | Compliance | Reference | Justification |
| --- | --- | --- | --- |
| 13-R01 The Licensing Fee for the four-year period is US$20,000, payable on an annual basis of US$5,000 per year. | Compliant | J.1.1 | US$5,000 per year / US$20,000 total Assumptions: [TO BE COMPLETED] |
| 13-R02 Licence | Compliant | J.1.2 | [TO BE COMPLETED] Assumptions: [TO BE COMPLETED] |
| 13-R03 Implementation | Compliant | J.1.3 | [TO BE COMPLETED] Assumptions: [TO BE COMPLETED] |
| 13-R04 Equipment | Compliant | J.1.4 | [TO BE COMPLETED] Assumptions: [TO BE COMPLETED] |
| 13-R05 Support / maintenance | Compliant | J.1.5 | [TO BE COMPLETED] Assumptions: [TO BE COMPLETED] |
| 13-R06 Optional services | Compliant | J.1.6 | [TO BE COMPLETED] Assumptions: [TO BE COMPLETED] |
| 13-R07 Recurring charges | Compliant | J.1.7 | [TO BE COMPLETED] Assumptions: [TO BE COMPLETED] |

# Appendices

## Appendix A. IWF responsibilities

The supplier acknowledges the IWF responsibilities stated in section 8.3 of the RFP.

Comments, if any
[TO BE COMPLETED]

DRAFTING NOTE - DELETE BEFORE SUBMISSION Before PDF export, replace all bracketed text, remove all drafting notes, and check that the proposal remains one coherent package.